# ME 320

Spring 2011

Project 1 The figure below shows a concept for an elliptical exercise bike. You need to design the bike to ensure that the motion can be completed within a comfortable range of motion for the user. Assume that the ankle does not move with respect to the lower leg and the footpad respectively. The following are suggested steps: - Identify the dimension of a 50 percentile male. - Find out the realistic ranges of motion for each relevant joint. - Draw the corresponding mechanism. - Identify the loops, the joints, of the bike including the rider. How many loops does it contain? - Calculate the mobility of the system. - Design the machine according to the Grashof conditions. Ensure that transmission angles are acceptable. - Design the bike to ensure that all motions of the joints of the rider remain within their acceptable ranges. - Use Working Model or Solidworks to present your final design.